## X350

### [edit] Specifications

#### [edit] Design and engineering

Like the Audi A8's, the X350's chassis and body are constructed from aluminium. While some steel is used in places throughout the chassis, the X350 has a stressed aluminium monocoque/chassis similar in general design to a conventional steel structure, but with two differences; its underbody components are bonded together with aerospace-grade epoxy adhesives while around 3,200 self-piercing rivets are used to create the new XJ's unibody.

This differs from the A8's construction which uses an aluminium spaceframe to which aluminium panels are then attached. However, the aluminium Audi A8 weighs a comparatively high 1,830 kg (4,000 lb) (3.0 V6 TDi) compared to 1,539 kg (3,390 lb) of the (3.0 V6) XJ. On its own, the current XJ's bodyshell weighs about the same as a contemporary BMW MINI. If the car were made of steel, it is estimated that it would weigh 40% more.

The new structure, and the need to continually improve the car's ride and handling, dictated a number of other mechanical changes. The third generation of Jaguar's rear suspension moved away from the previous double wishbone layout in favour of a more sophisticated multi-link arrangement.

Some of the styling features that distinguish the X350 from the previous XJ include the outer headlights, which are larger than the inner headlights, and wheels which are pushed out further towards the corners of the car (due to an increased wheelbase), both like the original Mark 1 XJ. Also, the X350 dispenses with the separate "sixthlight" rear side window of its predecessor, reverting back to two side windows with quarterlight glasses mounted in the rear doors, like the Mark 1. Moreover, beginning in 2004, changes have been made to the distinctive chrome side window frames of the XJ, where the use of chrome in the areas in between the front and back doors has been discontinued, in effect hiding the B-pillars. The curve in the rear door and rear screen resembles that of the Jaguar saloons of the 1950s and 1960s.

The vehicle was the first in Jaguar to be built using self-piercing rivets.[1]

#### [edit] Engines

| Model | Type | Power, torque@rpm |
| --- | --- | --- |
| 2.7 Litre V6 Diesel | 2.7L V6 twin turbo | 204 PS (150 kW; 201 hp), 435 N·m (321 lb·ft) |
| 3.0 Litre V6 Petrol | 3.0L V6 | 238 PS (175 kW; 235 hp) |
| 3.5 Litre V8 Petrol | 3.5L V8 | 258 PS (190 kW; 254 hp) |
| 4.2 Litre V8 Petrol | 4.2L V8 | 299 PS (220 kW; 295 hp) |
| 4.2 Litre V8 Petrol Supercharged | 4.2L V8 supercharged | 395 PS (291 kW; 390 hp) |

The V8 engines remained in the new model, but were now the revised and more powerful versions found in the 2003 S-Type. The 290 bhp 4.0 litre and 370 bhp 4.0 litre supercharged engines from the X308 Mk II were replaced by the S-Type's 294 bhp (219 kW; 298 PS) 4.2 L and 400 bhp (298 kW; 406 PS) 4.2 L supercharged units respectively, while a new 3.5  V8 was also introduced for the European market. Derived from the 3.2  engine of the previous model, it produced 265 bhp (198 kW; 269 PS) .

The 240 bhp (179 kW; 243 PS) 3.2 L V8 from the previous model was replaced by the 3.0L V6 from the X-Type and S-Type. The V6 powers the XJ6, while the XJ8 is powered by a naturally-aspirated V8. The XJR is powered by a supercharged 4.2 L V8. The XJ6 and the XJ TDVi are the only X350 models not sold in the Americas.

#### [edit] TDVi

In 2005 Jaguar introduced the diesel-powered XJ TDVi. The XJ TDVi features the same Ford-Peugeot-developed 2.7 litre twin-turbo V6 found in the S-Type. The model produces 204 bhp (152 kW; 207 PS) and 321 lb·ft (435 N·m) of torque, earning it praise as the most refined diesel engine to be found in any car, anywhere, with electronically controlled engine mounts in order to minimize vibration.

#### [edit] Daimler Super Eight/Super V8

The Super V8, also known as the Daimler Super Eight, is the most expensive model, with the XJR second. The Super V8, which debuted in the 2003 model year in the new X350 body style, was essentially a long-wheelbase, supercharged XJ8 with the more luxurious Vanden Plas, or Daimler interior. Its primary competitor was the Mercedes-Benz S55 AMG. A distinctive wire mesh grille and chrome-finished side mirrors set the Super V8 and the XJR apart from the less expensive XJ saloons. In 2005, the Super V8 model was replaced by the Daimler Super Eight in all markets other than North America. The Daimler Super Eight is essentially the same car, but with the addition of a different grille, boxwood inlays in the wood veneer and several other interior luxuries as standard. Daimler's US equivalent is no longer known as the Vanden Plas, but the Super V8. The Vanden Plas name is used on models that would be known as Sovereign elsewhere.

#### [edit] North American models

All North American XJs come standard with a 300 hp (224 kW) naturally aspirated engine. A 400 hp (298 kW) supercharged 4.2 L V8 engine is optional. The valvetrain has a dual overhead cam design with four valves per cylinder. The top speed is limited electronically to 155 mph (249 km/h).

#### [edit] Super V8 Portfolio

In early 2005, Jaguar announced its most exclusive and expensive XJ saloon since ceasing V12 production. Called the 2006 Super V8 Portfolio, it is a limited-edition trim level of the flagship Super V8 saloon. It debuted at the New York International Auto Show in March 2005, and is the most expensive Jaguar saloon produced to date, with a base price of US$115,995. The Portfolio features added power as well as exterior and interior enhancements, including a DVD player and 7-inch screens in the rear headrests. The Super V8 Portfolio, aimed at North American markets, became available in August 2005. It is available in only two new, limited colours: Black Cherry and Winter Gold.

The Super V8 Portfolio is powered by Jaguar's supercharged 400 hp (300 kW), 4.2 L, 32-valve, AJ-V8 engine. Top speed is 155 mph (249 km/h) and the Portfolio has a 0-60 mph acceleration time of under five seconds.

### [edit] Production

The last X350 vehicle was produced in March 2009.[2]